AUV Workbench Mission Report

Date of Mission	: Tuesday May 8th, 2007

Location of Mission	: Hunter Liggett

Vehicle(s) operated	: Scan Eagle

Pre-mission Operator	: William Harley, LT USN

Purpose of Mission	: Operation Test of digital streaming video and the ability to track and identify a moving ground target.

Waypoints and Mission:

Waypoint Map:

Summary of area and mission:

The UAV started by taking off and circling the airport.  When it was at sufficient altitude, it commenced the camera test.  The mission operator tasked the UAV to circle numerous locations in order to test its capability for the camera to track a user definable position and continuously stay locked onto that position while the UAV moved.  The test was performed with satisfactory results.

While performing the camera test, a range test was concurrently performed.  In this test the vehicle moved farther and farther away both in ground range and altitude to test signal strength at variable distances and to determine the maximum communications range using the “new/in testing” communication equipment.  The signal strength was within specifications for all distances tested.

The third test consisted using the UAV to track the Remus and allow the Remus to issue commands.  The Remus would take navigation control while the communications van retained control over the camera.  The camera would then be used to find, identify and track the Remus as it moved.  Due to the Remus needing software upgrades, a white government truck was used instead of the Remus.  Results of this test showed that the simulated Remus was able to drive the UAV but due to limited bandwidth the digital video quality was too jumpy and gritty to be able to identify and track the white truck on the light grey roads of Fort Hunter Liggett.

The mission continued with more camera tests until battery limitation parameters were met for vehicle recovery.

Waypoint list and details:

Key mission milestones at select waypoints:

Waypoint 17:  Commence testing video.  Operator in command and control van commenced camera testing.

Waypoint 37: Commence testing with the Remus.  The simulated Remus has navigational control.

Waypoint 43: Navigational Control returned to the command and control van.

Final Missions Trace:

From GPS Data				From Simulated Data

Missions Problems:

- 1) Remus unable to launch due to software reasons.

- 2) Poor resolution on digital video, preventing tracking of primary target.

Issues between Workbench and reality:

1) Circle command

The UAV was ordered to circle the southern point of the airport.  This point is represented by the circle on the bottom horizontal leg of the figure.  Notice that the simulation doesn’t have the UAV circle around it, but instead has it constantly try and hit the point.  A new function needs to be added to allow for circling around a point.

2) Consistency

The consistency of the wait command is unlike the real UAV.  Circling over a point quickly turns into a series of rotating rings, figure eights and numerous other figures shown below.  These patterns are unlike the UAV actual path.

AUV Workbench				GPS Data

a.

b.

c.

d.

Suggested Improvements:

Problem 1: Speed

A) Faster running of the program.

For my scenario it takes 5 – 10 minutes for the program to run through the ~90 minutes of real time movement.  For longer missions this time frame will become a limiting factor on the efficiency of making and evaluating missions.

B) Faster telemetry replay

For my scenario it takes ~20-30 minutes for the telemetry to replay.  This is unacceptable if you are just trying to look at the last 5 real-time minutes of the flight.

Resolution 1:

- Design a method in which the program generates all the data for the telemetry but does not show it on the screen. This process should take seconds instead of minutes.

- Instead of having the telemetry replay in the program, have it save the main geographic plot data in a video file.  This way the user can skip to the exact place that he wants to look at.  Additionally, since AUV workbench is for pre-mission, the telemetry can be easily uploaded into Power Point or any other medium and used as a briefing tool for the commander.  This would allow for additional versatility and ability to more easily brief people off site.  The current configuration assumes that all parties have a computer loaded with workbench, fully updated and connected to a briefing medium.  The probability the party who wants/needs to see the pre-mission of having the capabilities prepared and ready is unlikely at best.

Problem 2: Excess Commands

Many workbench commands such as loiter, set rudder angle, make turn rate, set elevator, set aileron, don’t seem to have any effect.  The next command automatically changes the command.

Resolution 2:

There should be a way to allow the user to fly without using waypoints.  Setting speed, direction, and control via rudders, ailerons and elevators.  This would allow finer control of numerous vehicles and a wider variety of uses.

Problem 3: Crashes

AUV Workbench crashes drop the workbench but leave secondary processes going in the background requiring a full reset of the system.  This is especially an issue when using commands such as loiter and composite waypoints in which the program isn’t always sure how to react, causing endless loops in the programming.

Resolution 3:

Check programming.  Add better instructions on the use of commands such as loiter and composite waypoints.

Problem 4: Autonomy

A high level mission based program code should be developed.  This would allow the programmer to use a waypoint and then to give a command such as “Conduct a circular search.”  The vehicle would automatically calculate all its waypoint and commands in order to do this mission.  This would remove the hours of waypoint adding it would take the user to create a continuously expanding circle, or similar higher level or complex mission that is typical to the missions in which a AUV would be used in today’s Navy.

Resolution 4:

Do research into video games that have similar top down controls as the workbench.  Investigate the independent AI these games use for conducting searches, patrols or other autonomous functions at the software levels and see what can be added to AUV Workbench.

Problem 5: Telemetry

The telemetry from the UAV took over 4 hours to play under initial configuration.  The GPS data collection starts once the vehicle is turned on, creating hours of meaningless data.  I went in and physically removed some of the latent data and reduced the time to about 45 minutes.  In addition, the data axis import were skewed, most likely due to the X,Y axis being X up/down, Y left/right.

Resolution 5:

Correct graphical display of imported telemetry and have the importer look for data in which nothing changes.