Shivok UW3302 FINAL MISSION REPORT

META DATA

Date:  18 May 2007

Time:  0700 Pier side - 0900 Mission Start

Location:  Monterey Bay Operations Area - vicinity of submerged pipe

Personnel:  Doug, John, Bill, Thad, 6 student observers

Test/report author:  Timothy Shivok (USW Applied Physics student)

Live/virtual:  Live

Environment:  At sea approximately 1 mile from shoreline.  Charted depth 30 meters.  Surface visibility=unrestricted, Sea visibility=2.5 meters.

Estimated sea state:  1-3 ft

Estimated set/drift:  060T - 1.5kts

Estimated wind:  220 @ 4kts

Estimated navigational fix accuracy:  UNK

Exercise objectives:  Videotape obstacle avoidance mission.  Students observe mission and produce model in AUV Workbench.

Mission:  Capture video of REMUS during obstacle avoidance mission run. Mission run objective is to maintain safe distance (20m stand-off) while over passing underwater buried pipe.

Hardware:  REMUS

Software:  Generic REMUS front end processor

Communications:  Standard configuration

Hardware issues

Hull, mechanical and propulsion:  NTR

Electrical and electronic:  Compass fault during initialization procedures

Sensors:  NTR

Communications:  NTR

Software issues

Versions:  21

Control:  NTR

Sensors:  NTR

Other:  NTR

Trouble log

Problem, workaround:  Compass fault during initialization procedures; compass calibration behavior programmed but set to "not accept previous calibration" behavior removed from scripting

Data collection

Mission narratives:  First of live test of obstacle avoidance with REMUS using autonomous image processing.

Data files retrieved:  Compass heading, current amps, days since 1970, depth, depth goal, dr latitude, dr longitude, estimated velocity, faults, faults 2, flags, gfi percent, goal latitude, goal longitude, heading goal, heading offset, heading rate, latitude, longitude, leg number, mission number, nav mode, objective index, objective number, obs fluorometer, percent cpu usage, pitch, pitch cmd, pitch fin position, pitch goal, pitch rate, roll, roll rate, rudder command, rudder fin position, software revision, spare slider 0, thruster cmd, thruster rpm, thruster rpm goal, vehicle pressure, vehicle temperature, volts.

Photographs and captions:  No video of REMUS available due to low visibility.  Side scan sonar images available in images folder within the project.  Special Sea Scan software for viewing is available through sonic marine website.

AUVW missions

Archive directory names:

Archive URL:

Mission plots

Matlab:  Latitude Longitude plot provided in project folder

AUVW:

Other:  Snapshot images of mission telemetry and script mission available in project folder.

Related missions

Mission:  Obstacle is rock bed surrounding sewer pipe. 10m wide rock bed 5m in height surrounds pipe.

Mission:   Underwater film camera unable to record images of REMUS – poor visibility.

Summary:  Obstacle avoidance mission was evaluated as satisfactory.  REMUS detected and maintained 10 meter stand-off distance from rock covered obstacle.  Video recording mission evaluated as unsatisfactory.  Future efforts to record REMUS will include a diver with a sonar reflector to more accurately determine range between REMUS and the camera.

The above portion of the report is identical to the Crisman & Hudson report for two reasons:

First, the REMUS mission being reported on is identical to the Crisman & Hudon mission as far as the META data is concerned. Crisman and Hudson emulated the data from the attached files REMUS_070518_1, while Jon and I emulated the REMUS_070518_2 mission.  The META data is the same for both.

Second, the mission I actually observed, the Camp Roberts Rascal mission, failed due to the Rascal’s propeller sheared off.  The Rascal was being flown by remote control and the operator noticed that the plane was acting strange. They landed the plane and were revving the throttle while someone held down the plane.  Next thing we knew, the propeller goes flying forward like a boomerang. Nobody was hurt, but needless to say, the mission was scrubbed. Therefore, I was assigned to work with LT Jon Morrison on the REMUS UUV Mission.

AUV WORKBENCH

Actual REMUS Telemetry data:

The actual data from the REMUS robot was given to us from Doug Horner in MATLAB form (state.mat).  We were unable to import the data into the AUV Workbench in that form. It appears that it is possible to import REMUS data into the Workbench, but it must be in a state.txt file format.  Everything we tried we could not get the data converted.

-recommend that Doug Horner provide the UW 3302 class with a MATLAB Script that will allow the conversion to the required text format or provide the data to the class in the txt format as well.  We prefer to keep the MATLAB format as well because as you can see above, it is a quick way to plot what the robot did and get a quick visualization without importing data into AUV Workbench. Most USW students are more proficient with MATLAB than they are with the AUV Workbench.

-None-the-less… a text version of the data was not supplied in a timely manor such that we could implement that data into our report.  However, we did implement a mission that was extremely close to ours for two reasons:

-One, we were to import telemetry data from the robot into AUV WorkBench as part of the class assignment. To learn the process and see visually how well we emulated the mission.

-Two, the mission we imported was close enough to our mission that it could show how there were\are a few problems that need to be resolved with the AUV WorkBench program.

AVCL Mission that approximately models the observed mission:

Telemetry from our emulated mission:

As you can see by comparing the below image to the above image; there are a few differences between the emulated mission and the actual telemetry, but they are very close.  It is slightly difficult to tell exactly how close they are though due to scales being different due to the failure to import the data directly into AUV Workbench.

Not too sure why REMUS data shows this jump.  It did not appear to really do this. Maybe it was a sensor error.

Not too sure how to emulate this final loop here. I tried to emulate by changing depth up to zero. However no loop

Telemetry from the imported mission:

Not too sure why REMUS data shows this jump.  It did not appear to really do this. Maybe it was a sensor error.

This was not in our mission. Ours ended up here.

Other Telemetry Related Plots:

Although, I did not get too much out the additional telemetry related plots, they are included as a feature in AUV Workbench and included in this report to show them for this mission.

Recommendations:

- I think that you should incorporate a cursor location overlay on the bottom of the 2D Mission Planner screen.

  - In addition, then add the ability to right click the mouse in order to add waypoint at the cursors location. It would make mission planning easier

- Snap to grid should be easier to turn on and off.  Or at least a display so that you can easily see what the current setting is on the screen.

- Telemetry import needs to be improved to be able to handle many different file types. Not just *.txt!

Recommended Class Improvements:

- Purchase an AUV or UUV for use by the class to program a mission, and collect the data from so that everyone can go and observe together and can gain learning from hearing others questions and answers as well as seeing multiple missions in one trip. Visit for one such AUV for sale http://www.procerusuav.com/productPricing.php

- Add a lab period to the class schedule and remove all the classroom lectures about the AUV Workbench from the class time and place them in the lab period. Way too much time was spent talking\debugging the AUV Workbench in class and very little of the book was covered. AUV Workbench is only a micron of depth into Autonomous systems!

- Get the small Lego Mindstorm Robots and during the lab period do the labs recommended by the author of the book to reinforce the lectures topics. See http://www.cse.usf.edu/~murphy/book/ for the labs and type in Lego Mindstorm Robots in Google for my places that sell the robot kits.