SMITH UW3302 FINAL MISSION REPORT

This is a mission report on the data from REMUS_070518_2, borrowed from Crisman and Hudson.  For the mission I tried to observe, a miscommunication in the meeting times resulted in a failure to meet.  It seems there is not enough data to go around, resulting in repetition.  The following META data is the same as the REMUS_070518_1 mission.

META DATA

Date:  18 May 2007

Time:  0700 Pier side - 0900 Mission Start

Location:  Monterey Bay Operations Area - vicinity of submerged pipe

Personnel:  Doug, John, Bill, Thad, 6 student observers

Test/report author:  Darin Smith

Live/virtual:  Live

Environment:  At sea approximately 1 mile from shoreline.  Charted depth 30 meters.  Surface visibility=unrestricted, Sea visibility=2.5 meters.

Estimated sea state:  1-3 ft

Estimated set/drift:  060T - 1.5kts

Estimated wind:  220 @ 4kts

Estimated navigational fix accuracy:  UNK

Exercise objectives:  Videotape obstacle avoidance mission.  Students observe mission and produce model in AUV Workbench.

Mission:  Capture video of REMUS during obstacle avoidance mission run. Mission run objective is to maintain safe distance (20m stand-off) while over passing underwater buried pipe.

Hardware:  REMUS

Software:  Generic REMUS front end processor

Communications:  Standard configuration

Hardware issues

Hull, mechanical and propulsion:  NTR

Electrical and electronic:  Compass fault during initialization procedures

Sensors:  NTR

Communications:  NTR

Software issues

Versions:  21

Control:  NTR

Sensors:  NTR

Other:  NTR

Trouble log

Problem, workaround:  Compass fault during initialization procedures; compass calibration behavior programmed but set to "not accept previous calibration" behavior removed from scripting

Data collection

Mission narratives:  First of live test of obstacle avoidance with REMUS using autonomous image processing.

Data files retrieved:  Compass heading, current amps, days since 1970, depth, depth goal, dr latitude, dr longitude, estimated velocity, faults, faults 2, flags, gfi percent, goal latitude, goal longitude, heading goal, heading offset, heading rate, latitude, longitude, leg number, mission number, nav mode, objective index, objective number, obs fluorometer, percent cpu usage, pitch, pitch cmd, pitch fin position, pitch goal, pitch rate, roll, roll rate, rudder command, rudder fin position, software revision, spare slider 0, thruster cmd, thruster rpm, thruster rpm goal, vehicle pressure, vehicle temperature, volts.

Photographs and captions:  No video of REMUS available due to low visibility.  Side scan sonar images available in images folder within the project.  Special Sea Scan software for viewing is available through sonic marine website.

Mission plots

Matlab:  Actual mission latitude and longitude

AUVW:  Emulated mission telemetry plots

Related missions

Mission:  Obstacle is rock bed surrounding sewer pipe. 10m wide rock bed 5m in height surrounds pipe.

Mission:   Underwater film camera unable to record images of REMUS – poor visibility.

Summary:  Obstacle avoidance mission was evaluated as satisfactory.  REMUS detected and maintained 10 meter stand-off distance from rock covered obstacle.  Video recording mission evaluated as unsatisfactory.  Future efforts to record REMUS will include a diver with a sonar reflector to more accurately determine range between REMUS and the camera.

Despite problems importing the actual mission data (provided in .mat format) into AUVW, the mission was successfully re-created and simulated.